# CH32V30x 嵌入式分层软件架构 — 完整方案（定稿版）

## 一、架构总览

### 1.1 目录结构

app_project/
│
├── BSW/                                    # 基础软件层
│   ├── Startup/                            # 启动汇编
│   ├── Core/                               # 内核支持 + 公共数据结构
│   │   ├── ringbuffer.h                    # 环形缓冲（无层属性，所有层可用）
│   │   └── ringbuffer.c
│   │
│   ├── MCAL/                               # 微控制器抽象层（芯片官方库，只读不改）
│   │   ├── inc/
│   │   │   ├── ch32v30x.h
│   │   │   └── ...
│   │   └── src/
│   │       ├── ch32v30x_gpio.c
│   │       └── ...
│   │
│   ├── Services/                           # 片内复杂驱动（非厂商标准库）
│   │   ├── drv_flash_eeprom.h
│   │   └── drv_flash_eeprom.c
│   │
│   ├── Board/                              # 板级：配置 + 初始化 + ISR + FIFO
│   │   ├── board_cfg.h                     # 唯一硬件配置窗口
│   │   ├── board_init.h
│   │   ├── board_init.c
│   │   ├── board_uart_fifo.h               # 通用串口 FIFO 引擎
│   │   ├── board_uart_fifo.c
│   │   └── ch32v30x_it.c                   # 中断服务（只推 FIFO，不碰 Device）
│   │
│   └── Device/                             # 板载外部器件驱动
│       └── esp8266/
│           ├── esp8266.h
│           └── esp8266.c
│
├── HAL/                                    # 硬件抽象接口层（APP 的唯一依赖）
│   ├── hal_common.h                        # 公共类型（HalStatus 等）
│   ├── hal_gpio.h
│   ├── hal_gpio.c
│   ├── hal_uart.h
│   ├── hal_uart.c
│   ├── hal_network.h
│   └── hal_network.c
│
├── Middleware/                             # 中间件（纯软件，当前预留）
│   └── .gitkeep
│
├── APP/                                    # 应用业务逻辑
│   ├── task_control.h
│   ├── task_control.c
│   ├── task_ota.h
│   └── task_ota.c
│
└── User/                                   # 顶层入口
    ├── main.c
    └── app_cfg.h

### 1.2 依赖关系图

┌─────────────────┐
                         │     APP 层       │
                         │  task_control   │
                         │    task_ota     │
                         └────────┬─────────┘
                                  │ 只调用 API_*()
                                  ▼
                         ┌─────────────────┐
                         │     HAL 层       │
                         │   hal_gpio ──────────→ MCAL (GPIO)
                         │   hal_uart ──────────→ MCAL (USART)
                         │   hal_network ───┐
                         └──────────────────┤
                                            │
              ┌─────────────────────────────┤
              ▼                             ▼
     ┌─────────────┐              ┌─────────────────┐
     │    MCAL     │              │   Device 层      │
     │ (芯片官方库)  │              │   esp8266        │
     └──────┬──────┘              └────────┬─────────┘
            │                              │
            │         ┌────────────────────┘
            │         │ (从 FIFO 读取接收数据)
            │         ▼
            │  ┌─────────────────┐         ┌──────────────────┐
            │  │   Board 层       │         │   Services 层     │
            │  │ board_uart_fifo │         │ drv_flash_eeprom │
            │  │       ▲         │         │   (片内复杂驱动)  │
            │  │       │         │         └────────┬─────────┘
            │  │ ch32v30x_it.c   │                  │
            │  │  (ISR 写 FIFO)  │                  │
            │  └─────────────────┘                  │
            │                                       │
            └───────────────────────────────────────┘
                               │
                       ┌───────┴───────┐
                       │ BSW/Core      │
                       │ ringbuffer    │ ← 纯数据结构，无层属性
                       └───────────────┘

三条硬规则：

- 零向上依赖： BSW 不引用任何 HAL 头文件。

- 零直接水平耦合： ISR 和 Device 之间只有 FIFO 数据接力，互不可见。

- 按需设层： 简单外设一跳到 MCAL，复杂驱动走 Services，外部器件走 Device。

### 1.3 配置窗口

| 文件 | 归属 | 职责 |
| --- | --- | --- |
| board_cfg.h | BSW/Board | 硬件级：引脚映射、时钟、UART 通道、FIFO 大小 |
| app_cfg.h | User | 应用级：版本号、WiFi SSID、服务器地址等业务参数 |

维护原则： 换板子只改 board_cfg.h。改业务参数只改 app_cfg.h。

## 二、基础层与抽象层实现 (BSW & HAL)

(注：此处保留了原始架构中的核心代码实现，并修复了部分截断问题)

### 2.1 - 2.19 配置、Core、Board、Services、Device、HAL 核心接口

(代码与初稿完全一致，已在项目中归档，此处重点展示补全的 Network 抽象层)

### 2.20 HAL/hal_network.c (修复与补全)

/**
 * @file    hal_network.c
 * @brief   网络抽象实现 — 内部绑定 BSW/Device/esp8266
 * @note    换 4G/以太网时只改这一个文件，hal_network.h 和 APP 不动
 */

#include "hal_network.h"
#include "esp8266.h"            /* 调用 Device 层（向下依赖） */
#include "board_init.h"
#include "board_cfg.h"
#include "app_cfg.h"

static NetState s_state = NET_IDLE;

HalStatus API_Net_Init(void)
{
    Board_InitEspUart(ESP_UART_BAUD);

    if (ESP8266_Init() != HAL_OK) {
        s_state = NET_ERROR;
        return HAL_ERROR;
    }
    
    s_state = NET_READY;
    return HAL_OK;
}

HalStatus API_Net_Connect(const char *ssid, const char *password, uint32_t timeout_ms)
{
    if (s_state == NET_ERROR) return HAL_ERROR;
    
    if (ESP8266_ConnectAP(ssid, password, timeout_ms) == HAL_OK) {
        return HAL_OK;
    }
    return HAL_TIMEOUT;
}

HalStatus API_Net_ConnectTCP(const char *ip, uint16_t port, uint32_t timeout_ms)
{
    if (s_state == NET_ERROR) return HAL_ERROR;

    if (ESP8266_TCPConnect(ip, port, timeout_ms) == HAL_OK) {
        s_state = NET_TCP_OPEN;
        return HAL_OK;
    }
    return HAL_TIMEOUT;
}

HalStatus API_Net_Send(const uint8_t *data, uint16_t len)
{
    if (s_state != NET_TCP_OPEN) return HAL_ERROR;
    
    /* 设置默认发送超时为 3000ms */
    return ESP8266_TCPSend(data, len, 3000); 
}

HalStatus API_Net_RxAvailable(uint8_t *available)
{
    if (!available) return HAL_INVALID_PARAM;
    *available = ESP8266_RxAvailable();
    return HAL_OK;
}

HalStatus API_Net_Read(uint8_t *buf, uint16_t max_len, uint16_t *actual_len)
{
    if (!buf || !actual_len) return HAL_INVALID_PARAM;
    *actual_len = ESP8266_Read(buf, max_len);
    return HAL_OK;
}

HalStatus API_Net_GetState(NetState *state)
{
    if (!state) return HAL_INVALID_PARAM;
    *state = s_state;
    return HAL_OK;
}

## 三、应用层实现 (APP & User)

这部分展示了应用层如何完全依赖 HAL 层和 app_cfg.h 进行业务开发，实现业务逻辑与底层硬件的彻底解耦。

### 3.1 APP/task_control.h

/**
 * @file    task_control.h
 * @brief   主控制业务逻辑
 */

#ifndef TASK_CONTROL_H
#define TASK_CONTROL_H

#include <stdint.h>
#include "hal_common.h"

void TaskControl_Init(void);
void TaskControl_Run(void); /* 置于主循环中 */

#endif /* TASK_CONTROL_H */

### 3.2 APP/task_control.c

/**
 * @file    task_control.c
 * @brief   主控制业务逻辑实现
 * @note    纯业务逻辑：只可见 API_ 开头的函数，绝对不包含 CH32 的任何头文件
 */

#include "task_control.h"
#include "hal_gpio.h"
#include "hal_uart.h"
#include "hal_network.h"
#include "app_cfg.h"

extern volatile uint32_t sys_tick_ms; /* 系统时基 */

static uint32_t s_last_100ms_tick = 0;
static uint32_t s_last_1000ms_tick = 0;

static void Process_100ms_Task(void)
{
    /* 按键检测业务 */
    ButtonState btnState;
    if (API_Button_GetState(BTN_KEY1, &btnState) == HAL_OK) {
        if (btnState == BTN_PRESSED) {
            API_Uart_Printf(UART_ID_DEBUG, "Button Pressed! Toggling Status LED.\r\n");
            API_Led_Toggle(LED_STATUS);
            API_Buzzer_Beep(50); /* 按键反馈音 */
        }
    }
}

static void Process_1000ms_Task(void)
{
    /* 网络状态监控与维持业务 */
    NetState net_state;
    API_Net_GetState(&net_state);
    
    if (net_state == NET_TCP_OPEN) {
        API_Led_SetStatus(LED_NET, LED_ON);
        /* 定时发送心跳 */
        API_Net_Send((const uint8_t *)"PING\r\n", 6);
    } else {
        API_Led_Toggle(LED_NET); /* 网络未就绪时闪烁 */
    }
}

static void Process_Network_Data(void)
{
    uint8_t has_data = 0;
    API_Net_RxAvailable(&has_data);
    
    if (has_data) {
        uint8_t rx_buf[64];
        uint16_t rx_len = 0;
        
        API_Net_Read(rx_buf, sizeof(rx_buf) - 1, &rx_len);
        if (rx_len > 0) {
            rx_buf[rx_len] = '\0';
            API_Uart_Printf(UART_ID_DEBUG, "NET RX: %s\r\n", rx_buf);
        }
    }
}

void TaskControl_Init(void)
{
    API_Uart_Printf(UART_ID_DEBUG, "App Init: Firmware v%s\r\n", APP_FW_VERSION);
    
    /* 尝试连接 WiFi 和 服务器 (阻塞式简易版，实际工程可改为状态机) */
    API_Uart_Printf(UART_ID_DEBUG, "Connecting to WiFi: %s...\r\n", APP_WIFI_SSID);
    if (API_Net_Connect(APP_WIFI_SSID, APP_WIFI_PASSWORD, 8000) == HAL_OK) {
        API_Uart_Printf(UART_ID_DEBUG, "WiFi Connected. Connecting to TCP...\r\n");
        if (API_Net_ConnectTCP(APP_SERVER_IP, APP_SERVER_PORT, 5000) == HAL_OK) {
            API_Uart_Printf(UART_ID_DEBUG, "TCP Connected!\r\n");
        }
    }
}

void TaskControl_Run(void)
{
    uint32_t current_tick = sys_tick_ms;

    /* 非阻塞：处理网络接收 */
    Process_Network_Data();

    /* 100ms 周期任务 */
    if (current_tick - s_last_100ms_tick >= APP_TASK_100MS_PERIOD) {
        s_last_100ms_tick = current_tick;
        Process_100ms_Task();
    }

    /* 1000ms 周期任务 */
    if (current_tick - s_last_1000ms_tick >= APP_TASK_1000MS_PERIOD) {
        s_last_1000ms_tick = current_tick;
        Process_1000ms_Task();
    }
}

### 3.3 User/main.c

/**
 * @file    main.c
 * @brief   系统顶层入口
 */

#include "ch32v30x.h"
#include "board_uart_fifo.h"
#include "hal_gpio.h"
#include "hal_uart.h"
#include "hal_network.h"
#include "drv_flash_eeprom.h"
#include "task_control.h"

volatile uint32_t sys_tick_ms = 0;

static void System_Initialization(void)
{
    /* 1. 内核时钟与中断初始化 */
    NVIC_PriorityGroupConfig(NVIC_PriorityGroup_2);
    SystemCoreClockUpdate();
    SysTick_Config(SystemCoreClock / 1000); /* 1ms SysTick */

    /* 2. BSW/Board 层基础设施初始化 */
    Board_UartFifo_Init();
    
    /* 3. BSW/Services 初始化 */
    FlashEEPROM_Init();

    /* 4. HAL 层初始化 */
    API_Uart_Init(UART_ID_DEBUG, 115200);
    API_Led_Init();
    API_Button_Init();
    API_Buzzer_Init();
    API_Net_Init();
}

int main(void)
{
    /* 硬件平台与抽象层初始化 */
    System_Initialization();
    
    /* 应用业务层初始化 */
    TaskControl_Init();

    /* 主循环 */
    while (1)
    {
        /* HAL 层底层轮询驱动（如蜂鸣器定时管理） */
        API_Buzzer_Update();

        /* 应用层任务调度 */
        TaskControl_Run();
    }
}